Jellyfin PS3 — Stutter & Freeze Debugging Log

All approaches attempted to fix video bouncing and freezing

# Current Status

- Pipeline runs at fps=19.9 (perfect 20fps)  → Working

- Double buffering fb=0,1,0,1 alternating correctly  → Working

- Zero network stalls throughout playback  → Working

- Video bouncing — frame N then frame N+1 then back to N  → NOT FIXED

- Audio completely silent  → NOT FIXED

# 1. Per-Frame Logging (fflush to HDD)

Root cause: Every plog() call triggers fflush() to the PS3 HDD taking 5-15ms. At 30fps with logging on every frame, this caused 6-7Hz regular stutter pattern.

- Removed flip.../flip done logging per frame  → Fixed stutter pattern

- Removed audio_s logging per frame  → Fixed stutter pattern

- Removed AU# submission logging (was firing on every IDR = every 4s)  → Fixed same-timestamp freezes

- Removed getPic FAIL logging (was firing 127,760 times per session!)  → Fixed major freezes

- Reduced heartbeat from every 200 to every 500 iterations  → Reduced HDD writes

# 2. Frame Timing

The frame timer was set to 30fps but expected_interval was 50ms (20fps), causing a mismatch.

- Changed timing_init(30,1) to timing_init(20,1)  → Fixed timing mismatch

- Confirmed expected_interval = 50000 matches 20fps  → Working

- Switched from sysGetCurrentTime syscall to direct TBR register read (__gettime())  → Reduced timing overhead to ~1ns

- Called sysGetTimebaseFrequency() at init instead of hardcoding 79.8MHz  → More accurate timing

- Resync threshold tightened from 3 frames to 1 frame  → Reduced catch-up lurching

# 3. RSX Framebuffer / Double Buffering

Multiple attempts to fix the frame-forward/back bouncing by changing buffer management.

- Tried color_buffer[curr_fb ^ 1] — writing to back buffer not front  → Fixed big freezes! Bouncing remains

- Tried single buffer mode (always buffer 0)  → Caused CRT scanlines, bouncing remained

- Added waitRSXIdle() after gcmSetWaitFlip  → Caused 1-3 second freezes — reverted

- Added waitflip() before blit  → Caused crash — reverted

- Confirmed fb=0,1,0,1 alternating correctly in logs  → Double buffering is correct

# 4. VDEC / Jitter Buffer

The PS3 VDEC runs on SPU hardware in parallel. Issues were in how frames were pulled from VDEC into the jitter buffer.

- Fixed OPD callback address — fn=0x000134a0 via __build_opd32  → VDEC callback now fires

- Added AU ring buffer (4 slots) — single static buffer was being overwritten async  → Fixed corrupt data

- SPS gating — only submit AUs after SPS NAL found  → Fixed INVALID_AU errors

- Changed VDEC_COLOR_MATRIX_BT709 to VDEC_COLOR_MATRIX_BT601  → Colour fix

- vdecGetPicItem called on every frame — caused 127k calls per session  → Caused freezes — now called once only

- Removed inner 8-poll loop — was causing producer/consumer deadlock  → Fixed first frame freeze

- Fixed TCP socket kept unread during 3s diagnostic wait — server closed connection  → Fixed disconnect after frame 1

- Pre-fill jitter buffer to 8 frames before display starts  → Smoother start

- Reduced jitter buffer from 24 to 8 slots  → Fixed permanently-full buffer

- Added sys_timer_usleep(1000) to cap loop at 1000 iter/sec  → Freed CPU for VDEC

# 5. Audio

Audio port opens successfully but no sound output throughout all testing.

- Fixed audio init order — audioGetPortConfig before audioPortStart  → No change to output

- Identified ri=1073807376 is readIndexAddr (pointer) not block index  → Diagnostic only

- Pre-filled 8 DMA blocks with silence at open time  → No crash but still silent

- Non-blocking poll with 1us timeout instead of blocking wait  → Removed audio stalls

# 6. Network / Stream

Jellyfin stream URL and HTTP chunked transfer investigation.

- Added stall detection — confirmed stalls=0 throughout (network is not the bottleneck)  → Network ruled out

- Removed MaxFramerate=20 from URL — Jellyfin rejected it with grey screen  → Reverted

- Kept stream.ts extension — ensures Jellyfin transcodes to TS not raw MKV  → Working

- Set socket timeout to 5ms so timing_frame_due() fires correctly  → Working

# 7. PTS Timestamps (B-Frame Reordering)

Current hypothesis: bouncing is caused by B-frames being displayed in decode order instead of display order because VDEC_TS_INVALID is passed for all PTS/DTS timestamps.

- Attempted PTS extraction from PES header and passing to vdecDecodeAu  → Built OK but more freezing — reverted

- Plan: try VideoProfile=baseline in Jellyfin URL to disable B-frames entirely  → NOT YET TRIED

# 8. Scaling & Display

Video output improvements made during development.

- Fixed tiling bug — row-by-row blit replacing flat memcpy  → Fixed tiled display

- Added nearest-neighbour scaling 1280x544 to 1920x1080  → Full screen video

- Precomputed sx[] and sy[] lookup tables — eliminated per-pixel division  → Eliminated ~2M divisions/frame

- Letterboxing for 2.35:1 content (1280x544 actual vs 1280x720 allocated)  → Correct aspect ratio

# 9. RSX FlexIO Blit Bottleneck

Root cause identified: the blit loop in player.cpp writes decoded frames pixel by pixel from jitter buffer (main RAM) into color_buffer (RSX VRAM) over the FlexIO bus. At 1280x720 this is 921,600 individual 32-bit writes per frame over a slow bus, on the PPU, 20 times per second. While the PPU is doing this it cannot check timing, read network, or do anything else — this is the remaining stutter source. SPU decode confirmed not the bottleneck (logs show queue depth 7-8, zero stalls throughout).

- Removed waitRSXIdle() from flip() — was causing full GPU pipeline stall every frame  → Partial improvement

- Confirmed cellVdec outputs ARGB32 directly — no YUV to RGB conversion needed on PPU  → SPU restructure not required

- Fix planned: allocate jitter buffer with rsxMemalign instead of memalign, replace blit loop with single rsxSetTransferImage call — RSX DMA pulls entire frame from registered RAM into VRAM without PPU involvement  → NOT YET IMPLEMENTED

# 10. Audio — Switched to minimp3 (Software PPU Decode)

Switched audio codec from AAC to MP3 in Jellyfin URL. Replaced cellADEC with minimp3 software decoder running on PPU. Audio port opens and DMA ring is populated but output sounds like a single cylinder motor — static cuts in and out in a repeating pattern.

- Switched to minimp3 header-only library for MP3 decode on PPU — no cellADEC header needed  → Decoding runs, audio still broken

- Suspected cause: readIndexAddr dereferenced as volatile u64* instead of volatile u32* — on PS3 big-endian, reading u64* at a u32 address shifts the value into high 32 bits giving garbage block indices 0-7  → NOT YET FIXED

- Possible sample rate mismatch — Jellyfin may be sending 44.1kHz MP3 but PS3 audio port configured for 48kHz, causing motor sound  → NOT YET INVESTIGATED

# Next Steps to Try

- Add &VideoProfile=baseline to Jellyfin URL — disables B-frames, zero code change

- If baseline fixes bounce: implement proper PTS extraction cleanly

- Fix audio block index: change volatile u64* to volatile u32* when dereferencing readIndexAddr

- Log info.hz from minimp3 on first decoded frame to confirm sample rate matches 48kHz

- Replace blit loop with rsxSetTransferImage — allocate jitter buffer with rsxMemalign, let RSX DMA the frame directly into VRAM

- Add waitflip() timeout in rsxutil.cpp to prevent RSX hang

- Set up git properly and push working build before any more changes

PS3 Jellyfin Homebrew Client — github.com/Tuggs-Bunny/JellyFin---PS3