Jellyfin PS3 Client

Pipeline Implementation Plan — Movian-Parity Roadmap

Based on code analysis of v1, v3, and v5 source releases

# 1. Overview

This document outlines every change required to bring the Jellyfin PS3 client to a stable, full-speed playback pipeline modelled on the Movian architecture. Changes are grouped by subsystem and ordered by priority — the threading and frame pacing fixes must land first as everything else depends on them.

Root cause summary: all three released versions share the same fundamental problem — the display loop is not paced to content frame rate. v1 had the correct infrastructure (timing_frame_due) but was single-threaded. v3 added a decode thread but never wired timing_frame_due into the display loop and set the decode thread at lower priority than display. v5 removed the timing system entirely.

# 2. Threading Model

The most important structural change. Currently v3 has 2 PPU threads; the target is 3 threads with strict ownership boundaries so no blocking call on one thread can stall another.

## 2.1  Add a dedicated Network thread

File: player.cpp — new thread function, e.g. network_thread_fn

Currently the network socket read (stream_read) lives inside the decode thread. When the socket times out or stalls, it blocks the decode thread from submitting AUs to VDEC. The fix is a dedicated thread whose only job is draining the socket into a ring buffer.

- Create a circular byte buffer (e.g. 256 KB) shared between the network thread and the decode thread.

- The network thread calls stream_read in a tight loop, writing into the ring buffer and sleeping 1ms when the buffer is full.

- The decode thread reads TS packets from the ring buffer instead of directly from the socket.

- This means waitflip() on the display thread can never stall the socket read.

sysThreadCreate(&net_tid, network_thread_fn, &net_ctx, 600, 64*1024, 0, "jf_network");

## 2.2  Fix decode thread priority

File: player.cpp — sysThreadCreate call for jf_decode

In v3 the decode thread is created at priority 1001. On PS3, lower number means higher priority. The main/display thread runs at approximately 1000, so the decode thread loses every scheduling contest. It needs to be higher priority than the display thread.

// Current — wrong

sysThreadCreate(&dec_tid, decode_thread_fn, &dec_ctx, 1001, 64*1024, 0, "jf_decode");

// Fixed

sysThreadCreate(&dec_tid, decode_thread_fn, &dec_ctx, 800, 128*1024, 0, "jf_decode");

Stack size is also increased from 64 KB to 128 KB — the decode thread processes TS packets, manages the jitter buffer, calls video_feed_ts and vdec_pull_frame, and does heartbeat logging, all of which push the 64 KB limit.

## 2.3  Add a dedicated Audio thread

File: player.cpp or audio.cpp — new thread function, e.g. audio_thread_fn

Currently audio_write_pcm is called from the display thread inside the blit block, while holding s_jbuf_mtx. A slow audio write blocks the decode thread from pushing frames for the entire duration. Audio should run on its own thread at a priority between network and display.

- Audio thread wakes on the audio event queue (sysEventQueueReceive) — already used in audio_silence().

- On each wake it writes the next PCM block and goes back to sleep.

- Display thread no longer calls audio_write_pcm at all.

sysThreadCreate(&aud_tid, audio_thread_fn, &aud_ctx, 900, 32*1024, 0, "jf_audio");

| Thread | Current Priority | Target Priority | Owns |
| --- | --- | --- | --- |
| Network thread | Does not exist | ~600 (highest) | Socket drain → ring buffer |
| Decode thread | 1001 (below display) | ~800 | Ring buffer → VDEC → jitter buffer |
| VDEC internal PPU | 1000 | 500 (via cfg) | H.264 decode on SPUs |
| Audio thread | On display thread | ~900 | PCM block writes |
| Display / main thread | ~1000 (default) | ~1000 (lowest) | Frame pace → blit → flip |

# 3. Frame Pacing

The single biggest bug present in all three versions. The display loop must consume frames at content frame rate, not at vsync rate (60 Hz) or audio event rate.

## 3.1  Wire timing_frame_due() into the display loop

File: player.cpp — the main display while loop

In v3, timing_init is called but timing_frame_due and timing_frame_shown are never used. The display loop runs at 60 Hz (one frame per waitflip), consuming jitter buffer slots 2.5x faster than a 24 fps decoder fills them. The jitter buffer drains immediately, jbuf_peek returns null most iterations, and duplicate/blank frames are flipped constantly.

The corrected display loop structure:

while (running && playing && !s_vdec_error) {

waitflip();

sysUtilCheckCallback();

// ... pad input ...

if (timing_frame_due()) {          // only consume at content rate

sysMutexLock(s_jbuf_mtx, 0);

const u8 *rslot = jbuf_peek();

sysMutexUnlock(s_jbuf_mtx);

if (rslot) {

// ... blit ...

sysMutexLock(s_jbuf_mtx, 0);

jbuf_pop();

sysMutexUnlock(s_jbuf_mtx);

frame_count++;

timing_frame_shown();      // advance the ticker

}

}

flip();

}

## 3.2  Detect content frame rate from PTS instead of hardcoding

File: player.cpp or video.cpp — vdec_submit or vdec_pull_frame

v3 calls timing_init(30, 1) hardcoded. The actual content coming from Jellyfin is 24 fps film. This means even if timing_frame_due were wired in correctly, frames would be consumed 25% faster than they are produced.

Movian detects frame rate from PTS deltas between the first few decoded frames. The simplest implementation is to measure the wall-clock interval between the first 8 frames returned by vdecGetPicture and use that to call timing_init with the detected rate before the display loop starts consuming.

- Alternatively, read the frame_rate field from the SPS NAL unit in vdec_submit — it is present in H.264 streams from Jellyfin.

- As a fallback, default to 24 rather than 30 — 24 fps film is far more common than 30 fps in a Jellyfin library.

// Minimum viable fix — change hardcoded rate

timing_init(24, 1);   // was timing_init(30, 1)

# 4. Jitter Buffer Mutex Scope

File: player.cpp — display loop blit block and video.cpp — jbuf_push

In v3, s_jbuf_mtx is held for the entire blit operation — jbuf_peek, the full pixel blit loop, audio_write_pcm, and jbuf_pop all run while the mutex is locked. The blit across a full 960x540 frame is the most expensive single operation in the player. While it runs, the decode thread cannot push new frames into the jitter buffer.

The mutex should only wrap the queue operations, not the blit:

// 1. Lock only to peek

sysMutexLock(s_jbuf_mtx, 0);

const u8 *rslot = jbuf_peek();

sysMutexUnlock(s_jbuf_mtx);   // release before blit

if (rslot) {

// blit runs without holding the mutex

for (u32 oy = 0; oy < sh; oy++) { ... }

// 2. Lock only to pop

sysMutexLock(s_jbuf_mtx, 0);

jbuf_pop();

sysMutexUnlock(s_jbuf_mtx);

}

Note: jbuf_peek must return a stable pointer for this to work safely — the slot must not be reused until jbuf_pop is called. This is already the case in the existing jitter buffer implementation.

# 5. VDEC Configuration

File: video.cpp — vdecConfig struct inside vdec_open

## 5.1  SPU count

Currently num_spus = 1. The PS3 CELL processor has 6 SPUs; homebrew can safely use up to 4. The VDEC firmware distributes H.264 decode work across all allocated SPUs — more SPUs means higher decode throughput. With the frame pacing fixed, the decoder will genuinely need to keep up with the display loop, making this change impactful at 720p.

cfg.num_spus = 4;   // was 1

Memory allocation must be scaled proportionally. The current code allocates attr.mem_size (returned by vdecQueryAttr) for 1 SPU. With 4 SPUs the allocation must be multiplied:

u32 mem_size_aligned =

((attr.mem_size * cfg.num_spus) + (1024*1024-1))

& ~(u32)(1024*1024-1);

If the allocation fails, fall back to 2 SPUs and log the result. Do not fall back silently.

## 5.2  VDEC PPU thread priority

Currently ppu_thread_prio = 1000, the same as the default main thread priority. The VDEC internal PPU thread coordinates SPU job submission and frame delivery. It should be higher priority than the display thread so frame callbacks are not delayed by display work.

cfg.ppu_thread_prio = 500;   // was 1000

# 6. Logging Improvements

The current log has no severity levels and no record of why playback stopped. These two additions make debugging significantly faster.

## 6.1  Log the reason playing becomes false

File: player.cpp — all sites that set playing = false

The log ends with show_player: loop exit running=1 playing=0 with no explanation. Every site that sets playing = false should log the reason before doing so:

// stream disconnected

plog("playing=0 reason=stream_eof"); playing = false;

// network error

plog("playing=0 reason=net_error"); playing = false;

// user pressed stop

plog("playing=0 reason=user_stop"); playing = false;

## 6.2  Log actual display FPS in heartbeat

File: player.cpp — heartbeat in decode thread

The heartbeat currently logs fr (total frame count) but not the rate at which frames are being displayed. Add a per-heartbeat frame counter and compute actual display FPS:

float display_fps = (frame_count - hb_fr_last) * 1000000.0f / (float)(hb_now - hb_last_us);

// include display_fps in the hb: log line

## 6.3  Detect and log audio clock stalls

File: player.cpp — audio thread or audio_write_pcm

The existing log shows aclk=0ms gaps — the audio clock returning zero intermittently. Add a consecutive-zero counter and log when it exceeds 2 frames:

if (aclk_us == 0) audio_stall_frames++;

else {

if (audio_stall_frames > 2) {

plog("WARN audio: clock stalled N frames");

}

audio_stall_frames = 0;

}

# 7. Implementation Order

Changes must be implemented in this order — each depends on the previous being stable.

- Frame pacing — wire timing_frame_due() into display loop and fix timing_init(24,1). This alone should restore correct playback speed.

- Decode thread priority — change from 1001 to 800 and increase stack to 128 KB.

- Jitter buffer mutex scope — narrow the lock to only wrap jbuf_peek and jbuf_pop, not the blit.

- VDEC configuration — bump to 4 SPUs, scale memory allocation, set PPU priority to 500.

- Network thread — extract stream_read into its own thread feeding a ring buffer.

- Audio thread — move audio_write_pcm off the display thread onto its own thread.

- Frame rate detection — replace hardcoded timing_init(24,1) with PTS-delta detection.

- Logging — add reason to playing=0, display FPS to heartbeat, audio stall detection.

# 8. Change Summary

| Component | Current State | Target State | Notes |
| --- | --- | --- | --- |
| Frame pacing | Not wired in display loop | timing_frame_due() used | Most critical fix |
| Content FPS | Hardcoded 30 fps | Detected or defaulted to 24 | Causes 25% drift |
| Decode thread priority | 1001 — below display | 800 — above display | v3 player.cpp |
| Decode thread stack | 64 KB | 128 KB | Prevent stack overflow |
| Network thread | Does not exist | New thread at priority 600 | Isolate socket reads |
| Audio thread | On display thread | New thread at priority 900 | Unblock display loop |
| Jitter buffer mutex | Held across full blit | Held only for peek/pop | video.cpp / player.cpp |
| VDEC SPU count | 1 SPU | 4 SPUs | video.cpp vdec_open |
| VDEC memory | attr.mem_size x1 | attr.mem_size x num_spus | video.cpp vdec_open |
| VDEC PPU priority | 1000 | 500 | video.cpp cfg |
| playing=0 reason | Not logged | Logged at every exit site | player.cpp |
| Display FPS heartbeat | Not logged | Logged per heartbeat interval | player.cpp |
| Audio stall detection | Not logged | Logged after 2 zero frames | player.cpp / audio.cpp |

Jellyfin PS3 Client — Pipeline Implementation Plan  |  Generated from source analysis of v1, v3, v5